BITSilicon Beginner Assignment

Digital Logic Warmup

- Decimal – 255.375

Binary – 11111111.011

Octal – 377.3

Hexadecimal – FF.6

- Decimal – 53.625

- Yes. Since 22n is always of the form 3k+1, and 22n+1 is always of the form 3m+2, a binary number with x ones at odd places and y even ones can be represented as 3t+x+2y. In the number 100111, there are 2 odd and 2 even ones, which is why it can be represented as 3t+2+4 = 3(t+6), and is divisible by 3.

- 11101001

- F = (A+B)(A’+C)(B+C’)

Opening the first two brackets, we get:

(AA’ + A’B + AC + BC)(B+C’)

AA’ = 0 and if both B and C are 1, then either A’B or AC will be 1, making the term BC redundant:

(A’B+AC)(B+C’) = (A’B+A’BC’+ABC+ACC’)

ACC’ = 0 and if A’BC’ = 1, A’B = 1 as well, making the term A’BC’ redundant:

(A’B+ABC) = B(A’ + AC)

If A = 0 and B = 1, F = 1. If A = 1, then C must be 1. This means we can change the term AC to just A without changing the logic of the statement, since when A = 1, AC = A, and when A = 0, F is always 1.

So, F = B(A’+C)

Therefore, F is 1 when B is 1 and either both A and C are 1 or A is 0.

- F(A,B,C) = C’

First Steps into Verilog

- Done

- module top_module (input a, input b, output out);

assign out = a&b;

endmodule

- module top_module (input a, input b, output out);

assign out = a|b;

endmodule

- module top_module (input a, input b, output out);

assign out = (a&(!b)|(!a)&b);

endmodule

Think Like a Designer

module top_module (input a, input b, input c, output out);

assign out = (a&b)|(b&c)|(a&c);

endmodule